Table of Contents

# Lite3 Orbit + Splat + AR — Lab Partner Handoff

Author: Ayaan Last updated: 2026-06-04 Purpose: Exact runbook for the same computers, containers, accounts, paths, and commands used in this lab’s setup.

## How to use this document

This is structured in three parts:

- Part I — Overview: read this first to get your bearings. Architecture, machine map, what files live where, and the small set of gotchas that will burn you if you don’t know them.

- Part II — Runbook: copy-paste exact procedures for boot, capture, Nav2, AR, splat training, and shutdown.

- Part III — Reference: lookup tables for UDP command codes, frame IDs, troubleshooting, and external docs.

When a command depends on which machine you’re on, the section header tells you. Commands are exact unless a line says to substitute a value.

# Part I — Overview

## 0. Read this first

Commands are written for the same accounts, Docker container names, ROS domains, paths, and network setup we used.

- Foxy robot/orbit/splat stack lives in Docker container foxy_ros on the NUC. Use ROS_DOMAIN_ID=0.

- Jazzy AR/iPhone/AprilTag stack lives in Docker container jazzy_ar on the NUC. Use ROS_DOMAIN_ID=42.

- The RealSense camera can only be owned by one stack at a time. Do not run Foxy capture and Jazzy AR camera simultaneously.

- The dog only obeys /cmd_vel after the dog-side navigation + move mode script is run. The mode session times out; rerun it before every motion session.

- If a Docker container already exists, do not run docker run again with the same name. Use docker exec or docker start.

## 1. System architecture

### 1.1 Topology diagram

┌─────────────────────┐
                │  Developer laptop   │
                └──┬──────────────┬───┘
                   │              │
                SSH│              │SSH/Tailscale
                   ▼              ▼
       ┌─────────────────┐   ┌───────────────────────┐
       │  Lite3 dog      │   │  Emolga (GPU)         │
       │  Jetson         │   │  100.110.138.76       │
       │  ysc@<dog-ip>   │   │  Nerfstudio,          │
       │                 │   │   splatfacto          │
       │  ~/start_       │   └───────────────────────┘
       │   transfer.sh   │              ▲
       │  publishes      │              │
       │   /leg_odom2    │              │ video pulled
       │                 │              │ from NUC via
       │  QNX motion     │              │ docker cp + scp
       │  host           │              │
       │  127.0.0.1:43893│   ┌──────────┴────────────────────┐
       └────────┬────────┘   │  NUC (locobot2)               │
                │            │  100.72.194.78                │
   Direct Eth   │            │                               │
   192.168.186.x│            │  ┌────────────┐ ┌───────────┐ │
                ▼            │  │ foxy_ros   │ │ jazzy_ar  │ │
       ┌────────────────┐    │  │ DOMAIN=0   │ │ DOMAIN=42 │ │
       │ dog eth1:      │────┼─▶│ Nav2,      │ │ AprilTag, │ │
       │ 192.168.186.2  │    │  │ orbit,     │ │ rosbridge │ │
       └────────────────┘    │  │ splat cap. │ │           │ │
                             │  │ RealSense* │ │RealSense* │ │
                             │  └────────────┘ └───────────┘ │
                             │   * mutually exclusive        │
                             │     time-share, NEVER both    │
                             └────────────┬──────────────────┘
                                          │ ARMLab/ARMLab-5G LAN
                                          │ (DDS multicast)
                                          ▼
                              ┌────────────────────────┐
                              │  AR base computer      │
                              │  native ROS2 Jazzy     │
                              │  DOMAIN=42             │
                              └──────────┬─────────────┘
                                         │ rosbridge ws
                                         ▼
                              ┌────────────────────────┐
                              │  iPhone (ARExplorer    │
                              │   app, MJPEG :8082)    │
                              └────────────────────────┘

The dog talks to the NUC over direct Ethernet for the ROS2 transfer stack. The QNX motion host inside the dog is reachable only from the dog itself at 127.0.0.1 — the Jetson does not route between the direct-Ethernet subnet and the motion host’s internal network.

Emolga is reached over Tailscale from the NUC and laptop. The AR base ↔ NUC link is on the ARMLab LAN, not Tailscale, because raw DDS multicast does not traverse Tailscale.

### 1.2 Machine map and exact identities

| Machine | Account / address | Purpose / exact notes |
| --- | --- | --- |
| NUC / locobot2 | ssh dog@100.72.194.78 | Main robot computer. Runs foxy_ros and jazzy_ar Docker containers. Tailscale IP confirmed as 100.72.194.78. |
| Dog / Lite3 Jetson | ssh ysc@192.168.0.129 OR ssh ysc@<current dog IP> | Dog-side stack. Runs ~/start_transfer.sh. Dog had wlan0=192.168.0.129/24 in one confirmed network state. If it changes, use current dog IP. |
| Dog direct Ethernet | dog eth1: 192.168.186.2 / NUC eno1: 192.168.186.3 | NUC-to-dog link. Ping from NUC to 192.168.186.2 worked. |
| Dog QNX motion host | 127.0.0.1:43893 from dog | Endpoint that accepted mode switch and speed commands. Use from dog only. |
| Emolga | ssh robodog@100.110.138.76 | GPU workstation for Nerfstudio / splatfacto. Tailscale IP confirmed as 100.110.138.76. |
| Base computer | native ROS2 Jazzy, ARMLab LAN | Runs iPhone/base side AR pipeline, RQT image viewer, rosbridge, and calibration tools. |
| iPhone | iphone_ip from app/HUD | Runs ARExplorer app. Connects to base rosbridge ws://<base-ip>:9090. Serves MJPEG at http://<iphone-ip>:8082/stream. |

### 1.3 Software file tree

NUC: /root/                            (inside foxy_ros container)
├── rplidar_ws_foxy/install/           # RPLIDAR A2M8 driver
├── lite3_ws/
│   ├── src/
│   │   ├── lite3_bringup/             # Nav stack + odom bridge
│   │   │   ├── lite3_bringup/
│   │   │   │   ├── odom_bridge_node.py    # /leg_odom2 → /odom + TF
│   │   │   │   └── motion_host_cli.py     # one-shot UDP commands
│   │   │   ├── launch/bringup.launch.py
│   │   │   ├── config/nav2_params.yaml    # CRITICAL: controller_server.odom_topic: /odom
│   │   │   ├── config/mapper_async.yaml
│   │   │   └── urdf/lite3_minimal.urdf.xacro
│   │   └── lite3_capture/             # Orbit + video capture
│   │       ├── lite3_capture/
│   │       │   ├── orbit_node.py
│   │       │   ├── motion_test_node.py
│   │       │   └── capture_orbit_node.py
│   │       ├── launch/orbit.launch.py
│   │       ├── launch/orbit_capture.launch.py
│   │       └── config/orbit_params.yaml   # radius, period, ramp, yaw_sign=-1
│   └── install/
└── captures/                          # video output (auto-created)

NUC: /home/dog/                        (on NUC host, outside containers)
├── jazzy_ar/Dockerfile                # Jazzy container build recipe
└── ar-robot-explorer/                 # cloned repo, mounted into jazzy_ar

Emolga: ~/                             (robodog@100.110.138.76)
├── splat_pipeline/
│   ├── splat_pipeline.sh
│   ├── fetch_and_splat.sh
│   └── README.md
├── gaussian_captures/                 # videos pulled from NUC
└── splat_runs/                        # per-run training outputs

## 2. Current status

| Component | Current state | What to do next |
| --- | --- | --- |
| Foxy dog/Nav2 | Structurally operational. Nav2 launch comes up and action server accepted goals. Physical Nav2 goal execution still needs final careful test. | Run dog mode switch immediately before sending Nav2 goal. Watch /cmd_vel. |
| Foxy /cmd_vel + orbit | Working. Dog moved after dog-side mode switch. Open-loop orbit was validated. | Use orbit_capture for splat capture only when dog is in clear area. |
| Foxy RealSense passthrough | Original blocker fixed by recreating foxy_ros with --privileged and -v /dev:/dev. /dev/video0..5 visible inside container. | Make sure Jazzy is not holding camera first. |
| Foxy orbit capture | Smoke-tested. A run recorded /root/captures/smoke_02_20260529_035742.mp4 and JSON with 336 frames at observed 29.7 fps. | Run full capture and then send video to Emolga. |
| Emolga splat pipeline | Scripts expected under ~/splat_pipeline; outputs under ~/splat_runs and ~/gaussian_captures. | Search for config.yml, .ply, .splat, .ksplat. Verify nerfstudio commands. |
| Jazzy AR image | Built. Packages RealSense, AprilTag, rosbridge verified. /camera/camera/color/image_raw publishes around 12-16 Hz with intermittent incomplete-frame warnings. | Run RQT on base, not inside Docker. Then verify /detections. |
| Base to NUC Jazzy DDS | Verified after moving NUC to ARMLab/ARMLab-5G network. Tailscale alone was not enough for raw DDS multicast. | Keep both on ARMLab for ROS2 DDS. Tailscale is fine for SSH/SCP. |
| iPhone AR | Next validation step. | Launch base pipeline with iphone_ip, run calibration_check and run_calibration, verify /ar_markers on iPhone. |

## 3. Golden rule: Foxy stack vs Jazzy stack

These two stacks should not be mixed. The same RealSense camera is shared by time, not by simultaneous access.

| Task | Container / machine | ROS domain | Camera owner |
| --- | --- | --- | --- |
| Dog Nav2 / orbit / splat video capture | foxy_ros on NUC | 0 | Foxy |
| AR / AprilTag / iPhone calibration | jazzy_ar on NUC + base computer | 42 | Jazzy |
| Nerfstudio training/export | Emolga | not ROS-dependent | none |

Before launching any RealSense driver, run this on the NUC host:

sudo lsof /dev/video0 2>/dev/null

If anything is listed, stop the stack currently holding the camera before proceeding.

## 4. Critical gotchas to know before editing

Five things that look incidental but will silently break the system if you touch them wrong. Read this section before modifying any config or source.

### 4.1 Nav2 DWB starvation bug

What it is. Nav2’s controller_server (running DWB) subscribes to a topic named by controller_server.odom_topic, which defaults to odom. If no nav_msgs/Odometry is being published on that topic, DWB silently starves: goals are accepted, the behavior tree runs, the planner produces a path, but no /cmd_vel ever leaves controller_server. No error is logged; it just doesn’t work.

The trap. Some Nav2 docs suggest setting bt_navigator.odom_topic. That parameter does not propagate to controller_server. Setting it alone does nothing useful.

What this repo does. The odom_bridge_node publishes both: 1. The TF odom → base_link 2. A nav_msgs/Odometry message on the /odom topic

And nav2_params.yaml explicitly sets:

controller_server:
  ros__parameters:
    odom_topic: /odom

Implications for editing. - If you refactor odom_bridge_node, the /odom Odometry topic must remain (not just TF). - If you edit nav2_params.yaml, the controller_server.odom_topic line must stay set to /odom. - If goals stop executing despite being accepted, check ros2 topic hz /odom first. If it’s zero, that’s the problem.

### 4.2 Foxy robot_description must be wrapped in ParameterValue

What it is. When bringup.launch.py passes a URDF to robot_state_publisher via Command(['xacro ', urdf_file]), Foxy’s launch_ros tries to YAML-parse the resulting XML as a parameter value. Any colon inside an XML comment (e.g. “What this file provides:”) gets interpreted as a YAML mapping and the launch fails with a yaml.scanner.ScannerError.

The fix. Wrap the Command output explicitly as a string:

from launch_ros.parameter_descriptions import ParameterValue

parameters=[{
    'robot_description': ParameterValue(
        Command(['xacro ', urdf_file]),
        value_type=str,
    ),
}]

Galactic+ handles this automatically. Foxy does not.

Implication for editing. If you replace the URDF or change the robot_state_publisher block, keep the ParameterValue wrapper. If you see “Unable to parse the value of parameter robot_description as yaml”, this is the cause.

### 4.3 Navigation mode session timeout

The dog’s QNX motion host requires heartbeats at ≥2 Hz to maintain a control session. The dog-side mode switch script (Appendix A.1) sends heartbeats, then mode commands, then more heartbeats, then exits. The session expires shortly after heartbeats stop.

Practical consequence. Between two consecutive /cmd_vel sessions, the dog will silently revert to manual mode. If you ran the mode switch ten minutes ago and the dog won’t move, run it again. This is normal.

### 4.4 RealSense is a single-owner USB device

The Foxy container and the Jazzy container cannot both run the RealSense driver at the same time. There is no software enforcement of this — it’s operational discipline. If you accidentally start both, you’ll get cryptic device-busy errors. Always check sudo lsof /dev/video0 before launching a camera node.

### 4.5 Browser-rename gotcha

When downloading the same zip filename multiple times, browsers append (1), (2), etc. instead of overwriting. This bit us twice — once with lite3_bringup.zip and once with lite3_capture_v2.zip. The receiving machine ended up with a stale file under the expected name and a new file under <name> (1).zip.

Defense. Before scp-ing a file you just downloaded, verify the contents:

python3 -c "
import zipfile
z = zipfile.ZipFile('/home/robodog/Downloads/<file>.zip')
print('files:', len(z.namelist()))
"

If renaming for clarity, use _v2, _v3 suffixes deliberately, and scp with the explicit filename as the destination:

scp "/home/robodog/Downloads/file (1).zip" nuc:~/file_v2.zip

# Part II — Runbook

## 5. NUC boot / reconnect and container basics

### 5.1 Reconnect to the NUC

ssh dog@100.72.194.78

Check you are on the right machine:

hostname
ip -br addr
sudo docker ps -a

### 5.2 If NUC Wi-Fi is wrong, switch to ARMLab

This was required for Jazzy DDS between base and NUC. SSH may drop; reconnect with Tailscale after switching.

nmcli connection show

sudo nmcli connection up "ARM_LAB" ifname wlp0s20f3
# If that does not work:
sudo nmcli connection up "ARMLab-5G" ifname wlp0s20f3

# Then reconnect:
ssh dog@100.72.194.78

### 5.3 Container name conflict fix

If you see Conflict. The container name "/jazzy_ar" is already in use, use exec/start, not docker run.

sudo docker ps -a | grep jazzy_ar

# If running:
sudo docker exec -it jazzy_ar bash

# If Exited:
sudo docker start jazzy_ar
sudo docker exec -it jazzy_ar bash

# If you really want to recreate it:
sudo docker stop jazzy_ar
sudo docker rm jazzy_ar

## 6. Foxy robot / orbit / splat workflow

### 6.1 Start dog-side transfer stack

On laptop/base terminal, SSH into the dog and leave this running:

ssh ysc@192.168.0.129
# If that IP is unavailable, use the current dog IP:
# ssh ysc@<dog-ip>

~/start_transfer.sh

This brings up the dog-side stack that publishes /leg_odom2 and runs motion_sender subscribing to /cmd_vel.

### 6.2 Enter foxy_ros and source exact setup

On NUC host:

ssh dog@100.72.194.78
sudo docker start foxy_ros
sudo docker exec -it foxy_ros bash

Inside foxy_ros:

export ROS_DOMAIN_ID=0
export ROS_LOCALHOST_ONLY=0
source /opt/ros/foxy/setup.bash
source /root/rplidar_ws_foxy/install/setup.bash
source /root/lite3_ws/install/setup.bash

Optional one-time convenience inside foxy_ros to skip the export/source lines on every shell:

cat >> /root/.bashrc << 'EOF'
# ROS2 + Lite3 workspaces
export ROS_DOMAIN_ID=0
export ROS_LOCALHOST_ONLY=0
source /opt/ros/foxy/setup.bash
source /root/rplidar_ws_foxy/install/setup.bash
source /root/lite3_ws/install/setup.bash 2>/dev/null
EOF

### 6.3 Preflight checks inside foxy_ros

ros2 topic hz /leg_odom2
ls /dev/ttyUSB0
ls /dev/video*
ros2 pkg executables lite3_bringup
ros2 pkg executables lite3_capture

Expected: /leg_odom2 around 180-200 Hz, LiDAR at /dev/ttyUSB0, and RealSense devices /dev/video0 through /dev/video5 if Foxy owns camera.

### 6.4 If Foxy does not see RealSense, recreate foxy_ros exactly this way

This is the exact fix that preserved the existing workspace and gave the container access to /dev/video*. Run on NUC host, not inside container.

sudo docker commit foxy_ros foxy_ros_with_ws:latest
sudo docker stop foxy_ros
sudo docker rename foxy_ros foxy_ros_old

sudo docker run -it \
  --name foxy_ros \
  --network host \
  --ipc host \
  --privileged \
  -v /dev:/dev \
  foxy_ros_with_ws:latest \
  bash

Inside the recreated container, source the Foxy setup and verify:

source /opt/ros/foxy/setup.bash
source /root/rplidar_ws_foxy/install/setup.bash
source /root/lite3_ws/install/setup.bash
ls /dev/video*

Rollback if needed:

sudo docker stop foxy_ros
sudo docker rm foxy_ros
sudo docker rename foxy_ros_old foxy_ros
sudo docker start foxy_ros

### 6.5 Run the dog-side mode switch immediately before motion

Open a second terminal to the dog and run Appendix A.1. Do this right before /cmd_vel, Nav2, or orbit. The session expires after heartbeats stop (see §4.3).

### 6.6 Exact /cmd_vel pulse that worked

Inside foxy_ros after dog is in navigation + move mode:

timeout 1 ros2 topic pub -r 10 --qos-reliability best_effort /cmd_vel geometry_msgs/msg/Twist \
'{linear: {x: 0.04, y: 0.0, z: 0.0}, angular: {x: 0.0, y: 0.0, z: 0.0}}'

ros2 topic pub --once --qos-reliability best_effort /cmd_vel geometry_msgs/msg/Twist \
'{linear: {x: 0.0, y: 0.0, z: 0.0}, angular: {x: 0.0, y: 0.0, z: 0.0}}'

Important: paste the full ros2 command. Do not paste only the YAML body; the shell will report command not found.

### 6.7 Exact orbit + RealSense capture command that worked

Inside foxy_ros, after dog stack is running and dog-side mode switch has just been run:

sleep 5 && ros2 launch lite3_capture orbit_capture.launch.py basename:=smoke_02

Observed successful output from one smoke run: RealSense D435I detected, color stream RGB8 640x480 @ 30 fps, recording to /root/captures/smoke_02_20260529_035742.mp4, finalized with 336 frames and observed 29.7 fps.

After launch exits or after Ctrl-C, check output:

ls -lh /root/captures/smoke_02_*.mp4
cat /root/captures/smoke_02_*.json

### 6.8 Full 360° orbit parameter edit

The default orbit settings give only about 132° of arc, not a full revolution. The math:

total_angle = (2π / orbit_period_s) × (ramp_seconds + orbit_duration_s)

With defaults period=60, duration=20, ramp=2: (2π/60) × 22 = 2.30 rad ≈ 132°.

(Why ramp + duration and not 2 × ramp + duration: the trapezoidal envelope’s ramps each integrate to half their full-speed arc, so two 2-second ramps together cover the same arc as one 2-second full-speed segment.)

For near full 360°, edit orbit params inside foxy_ros:

nano /root/lite3_ws/src/lite3_capture/config/orbit_params.yaml

Set:

orbit_period_s: 30.0
orbit_duration_s: 28.0
ramp_seconds: 2.0
yaw_sign: -1

Verify: (2π/30) × 30 = 2π ≈ 360°. ✓

Because symlink install is used for config, no rebuild is normally needed. Launch with a longer capture duration if the launch arg exists:

ros2 launch lite3_capture orbit_capture.launch.py \
  basename:=full_orbit_01 \
  capture_duration_s:=36

Drift caveat. At 0.75 m radius without closed-loop feedback, radius wanders 10-20 cm over a full 360°. Fine for first-pass splats; the reconstruction will show fuzzier results at the perimeter than a perfectly circular orbit would.

## 7. Nav2 physical execution test

This is independent of the splat demo. Use this only when the dog is in a safe open area.

- Start dog stack: ssh into dog and run ~/start_transfer.sh.

- Enter foxy_ros and source Foxy/lite3 setup.

- Run Appendix A.1 on dog immediately before sending the goal.

- Launch Nav2 inside foxy_ros:

- ros2 launch lite3_bringup bringup.launch.py use_nav2:=true

- In a second foxy_ros shell, verify odom and TF:

- ros2 topic info -v /odom
ros2 topic hz /odom
timeout 5 ros2 run tf2_ros tf2_echo map base_link
ros2 action list | grep navigate_to_pose

- In a third foxy_ros shell, watch the command output:

- ros2 topic echo /cmd_vel

- Send the small goal:

- ros2 action send_goal /navigate_to_pose nav2_msgs/action/NavigateToPose \
"{pose: {header: {frame_id: 'map'}, pose: {position: {x: 0.3, y: 0.0, z: 0.0}, orientation: {w: 1.0}}}}"

Expected: goal accepted, /cmd_vel streams, dog physically moves. If accepted but no movement, rerun the dog mode switch (Appendix A.1) and retry immediately.

If /cmd_vel stays silent even after accepting the goal, that’s the DWB starvation symptom from §4.1 — check ros2 topic hz /odom and the controller_server.odom_topic setting.

## 8. Emolga Nerfstudio / splat workflow

### 8.1 Connect and verify nerfstudio

ssh robodog@100.110.138.76
conda activate nerfstudio
which ns-process-data ns-train ns-export
ns-train --help | head

### 8.2 Search for generated Nerfstudio / splat outputs

Use this to find exactly what already exists on Emolga:

find ~/splat_runs ~/outputs ~ -type f \( \
  -name "config.yml" -o \
  -name "transforms.json" -o \
  -name "dataparser_transforms.json" -o \
  -iname "*.ply" -o \
  -iname "*.splat" -o \
  -iname "*.ksplat" \
\) -printf "%TY-%Tm-%Td %TH:%TM %s %p\n" 2>/dev/null | sort | tail -100

Find the latest Nerfstudio training config:

find ~ -path "*/outputs/*/splatfacto/*/config.yml" \
  -printf "%TY-%Tm-%Td %TH:%TM %p\n" 2>/dev/null | sort | tail -20

CONFIG=$(find ~ -path "*/outputs/*/splatfacto/*/config.yml" 2>/dev/null | sort | tail -1)
echo "$CONFIG"

### 8.3 View / export latest Nerfstudio run

ns-viewer --load-config "$CONFIG"

Export Gaussian splat:

mkdir -p ~/splat_exports/latest

ns-export gaussian-splat \
  --load-config "$CONFIG" \
  --output-dir ~/splat_exports/latest

find ~/splat_exports ~/splat_runs ~ -type f \( -iname "*.ply" -o -iname "*.splat" -o -iname "*.ksplat" \) \
  -printf "%TY-%Tm-%Td %TH:%TM %s %p\n" 2>/dev/null | sort | tail -50

### 8.4 End-to-end orchestrator command

Once orbit_capture works, run this on Emolga. This command is the intended one-command path from capture to splat.

~/splat_pipeline/fetch_and_splat.sh demo_01 28

It SSHes to NUC, runs orbit_capture in foxy_ros, copies MP4+JSON out, SCPs to Emolga, then runs ns-process-data, ns-train splatfacto, and ns-export. It assumes the dog is already in navigation/move mode.

## 9. Jazzy AR / iPhone workflow

### 9.1 Build image — already done, but exact command is here

On NUC host:

mkdir -p ~/jazzy_ar
cd ~/jazzy_ar

cat > Dockerfile << 'EOF'
FROM ros:jazzy
RUN apt-get update && apt-get install -y \
    ros-jazzy-apriltag-ros \
    ros-jazzy-rosbridge-server \
    ros-jazzy-realsense2-camera \
    ros-jazzy-tf2-tools \
    python3-scipy \
    python3-colcon-common-extensions \
    python3-opencv \
    git lsof \
    && rm -rf /var/lib/apt/lists/*
WORKDIR /ros2_ws
CMD ["bash"]
EOF

sudo docker build -t jazzy_ar .

Verify:

sudo docker run --rm jazzy_ar bash -c \
  "source /opt/ros/jazzy/setup.bash && ros2 pkg list | grep -E 'realsense|apriltag|rosbridge'"

Expected packages: apriltag_msgs, apriltag_ros, realsense2_camera, realsense2_camera_msgs, rosbridge_library, rosbridge_msgs, rosbridge_server.

### 9.2 Load ar-robot-explorer repo — exact fix if folder is bad

We hit a case where ~/ar-robot-explorer existed but was not a git repo. Fix on NUC host:

cd ~
mv ar-robot-explorer ar-robot-explorer_old_$(date +%Y%m%d_%H%M%S)
git clone https://github.com/ahmedm-02/ar-robot-explorer.git
cd ~/ar-robot-explorer
git remote -v
ls ar_explorer/package.xml
ls ar_explorer/setup.py

### 9.3 Start / restart jazzy_ar exactly

If recreating from scratch:

sudo docker stop jazzy_ar
sudo docker rm jazzy_ar

sudo docker run -it --name jazzy_ar \
  --network host \
  --privileged \
  -v /dev:/dev \
  -v ~/ar-robot-explorer/ar_explorer:/ros2_ws/src/ar_explorer \
  -e ROS_DOMAIN_ID=42 \
  jazzy_ar bash

If it already exists, use:

sudo docker start jazzy_ar
sudo docker exec -it jazzy_ar bash

### 9.4 Build / source ar_explorer inside jazzy_ar

source /opt/ros/jazzy/setup.bash
export ROS_DOMAIN_ID=42
ls /dev/video*

cd /ros2_ws
colcon build --packages-select ar_explorer
source install/setup.bash

ros2 pkg list | grep ar_explorer

Expected /dev/video0..5 if RealSense is plugged into NUC and container has /dev mount.

### 9.5 Launch NUC-side AR camera / AprilTag pipeline

export ROS_DOMAIN_ID=42
source /opt/ros/jazzy/setup.bash
source /ros2_ws/install/setup.bash

ros2 launch ar_explorer ar_pipeline.launch.py machine:=dog

Known warning: RealSense may print Incomplete video frame detected / Frame Corrupted. Continue if the image topic publishes and detections work.

### 9.6 In a second jazzy_ar shell, verify image and detections

sudo docker exec -it jazzy_ar bash

Inside:

export ROS_DOMAIN_ID=42
source /opt/ros/jazzy/setup.bash
source /ros2_ws/install/setup.bash

ros2 topic list | grep -E "camera|image|detections"
ros2 topic hz /camera/camera/color/image_raw

Observed image rate was about 12-16 Hz after USB 3 replug. That is lower than ideal but enough for smoke testing. Then put AprilTag 17 in view:

ros2 topic echo /detections --once
ros2 topic info /detections -v

# Optional TF check if tag frame exists:
timeout 5 ros2 run tf2_ros tf2_echo camera_color_optical_frame tag_17

### 9.7 RQT image viewer — do this on base, not inside Docker

Inside jazzy_ar, rqt_image_view failed with “could not connect to display” because the Docker container had no GUI display. Run RQT on the base computer instead.

On base computer:

export ROS_DOMAIN_ID=42
source /opt/ros/jazzy/setup.bash

ros2 topic list | grep image
ros2 topic hz /camera/camera/color/image_raw

sudo apt update
sudo apt install -y ros-jazzy-rqt-image-view

ros2 run rqt_image_view rqt_image_view

In the RQT dropdown, select /camera/camera/color/image_raw.

### 9.8 Base-side iPhone pipeline

On base computer, after NUC-side Jazzy pipeline is running and iPhone app is open. Replace <iphone-ip> with the IP shown in the iPhone app/HUD.

export ROS_DOMAIN_ID=42
source /opt/ros/jazzy/setup.bash
source ~/ros2_ws/install/setup.bash

ros2 launch ar_explorer ar_pipeline.launch.py \
  machine:=base \
  iphone_ip:=<iphone-ip>

In another base terminal, confirm base sees NUC detections:

export ROS_DOMAIN_ID=42
source /opt/ros/jazzy/setup.bash
source ~/ros2_ws/install/setup.bash

ros2 topic list | grep detections
ros2 topic echo /detections --once

### 9.9 Calibration and iPhone marker smoke test

Put AprilTag 17 where both RealSense and iPhone can see it. On base:

ros2 run ar_explorer calibration_check
ros2 run ar_explorer run_calibration
ls -lh ~/.ros/ar_explorer_calibration.json
ros2 topic echo /ar_markers --once
ros2 run ar_explorer ar_marker_publisher

Expected: calibration JSON is written, /ar_markers publishes, and a marker appears in iPhone AR view.

## 10. DDS / network verification

Use this if the base cannot see NUC topics. Both machines must be on ARMLab/ARMLab-5G or another multicast-friendly LAN. Tailscale is not enough for raw DDS discovery.

On NUC host, listener:

sudo docker run -it --rm --network host -e ROS_DOMAIN_ID=42 ros:jazzy bash -lc \
"apt update >/dev/null && apt install -y ros-jazzy-demo-nodes-cpp >/dev/null && source /opt/ros/jazzy/setup.bash && ros2 run demo_nodes_cpp listener"

On base computer, talker:

export ROS_DOMAIN_ID=42
source /opt/ros/jazzy/setup.bash
ros2 run demo_nodes_cpp talker

Expected NUC output:

[listener]: I heard: [Hello World: 12]
[listener]: I heard: [Hello World: 13]
[listener]: I heard: [Hello World: 14]

## 11. Shutdown procedure

- Stop ROS launches first with Ctrl-C in foxy_ros and/or jazzy_ar.

- Return the dog to manual/controller mode using Appendix A.2 on the dog.

- Then Ctrl-C ~/start_transfer.sh on the dog.

- Optionally stop containers on NUC host:

- sudo docker stop jazzy_ar
sudo docker stop foxy_ros

## 12. Quick task recipes

### 12.1 AR camera smoke test only

# NUC host
ssh dog@100.72.194.78
sudo docker start jazzy_ar
sudo docker exec -it jazzy_ar bash

# inside jazzy_ar
export ROS_DOMAIN_ID=42
source /opt/ros/jazzy/setup.bash
source /ros2_ws/install/setup.bash
ros2 launch ar_explorer ar_pipeline.launch.py machine:=dog

# second terminal into jazzy_ar
sudo docker exec -it jazzy_ar bash
export ROS_DOMAIN_ID=42
source /opt/ros/jazzy/setup.bash
source /ros2_ws/install/setup.bash
ros2 topic hz /camera/camera/color/image_raw
ros2 topic echo /detections --once

### 12.2 Splat capture only

# Dog terminal
ssh ysc@192.168.0.129
~/start_transfer.sh

# Dog second terminal: run Appendix A.1 mode switch

# NUC / Foxy terminal
ssh dog@100.72.194.78
sudo docker start foxy_ros
sudo docker exec -it foxy_ros bash

export ROS_DOMAIN_ID=0
export ROS_LOCALHOST_ONLY=0
source /opt/ros/foxy/setup.bash
source /root/rplidar_ws_foxy/install/setup.bash
source /root/lite3_ws/install/setup.bash

sleep 5 && ros2 launch lite3_capture orbit_capture.launch.py basename:=smoke_02

# Check output
ls -lh /root/captures/smoke_02_*.mp4
cat /root/captures/smoke_02_*.json

### 12.3 Latest splat on Emolga only

ssh robodog@100.110.138.76
conda activate nerfstudio

find ~/splat_runs ~/outputs ~ -type f \( \
  -name "config.yml" -o -iname "*.ply" -o -iname "*.splat" -o -iname "*.ksplat" \
\) -printf "%TY-%Tm-%Td %TH:%TM %s %p\n" 2>/dev/null | sort | tail -100

CONFIG=$(find ~ -path "*/outputs/*/splatfacto/*/config.yml" 2>/dev/null | sort | tail -1)
echo "$CONFIG"

ns-viewer --load-config "$CONFIG"

# Part III — Reference

## 13. Troubleshooting — exact errors we saw

| Symptom | Meaning | Exact fix |
| --- | --- | --- |
| docker: Conflict. The container name "/jazzy_ar" is already in use | Container already exists. | sudo docker exec -it jazzy_ar bash, or sudo docker start jazzy_ar first if stopped. |
| could not connect to display from rqt_image_view inside Docker | Container was started without GUI DISPLAY/X11. | Run rqt_image_view on base computer instead. |
| Incomplete video frame detected / Frame Corrupted | RealSense stream has dropped/corrupt frames, usually USB bandwidth/cable/port. | Use USB 3, check ros2 topic hz. Continue if stream stays alive and detections work. |
| /cmd_vel publishes but dog does not move | Dog not in navigation + move mode, or mode session expired. | Rerun Appendix A.1 on dog immediately before the motion command. |
| Base cannot see NUC Jazzy topics | DDS multicast not working across networks/Tailscale. | Put NUC and base on ARMLab/ARMLab-5G and rerun demo_nodes_cpp test. |
| No /dev/video* inside container | Container lacks /dev passthrough or camera not plugged at start. | Recreate with --privileged -v /dev:/dev; verify host sees /dev/video* first. |
| Shell says command not found after pasting Twist YAML | Only YAML was pasted, not the ros2 topic pub command. | Paste the complete ros2 topic pub command including message type and quoted YAML. |
| Nav2 goal accepted but no Twists | DWB starving on /odom (see §4.1). | Verify controller_server.odom_topic: /odom, ros2 topic hz /odom, and /cmd_vel echo. |
| Unable to parse the value of parameter robot_description as yaml | Foxy YAML-parsing URDF (see §4.2). | Wrap with ParameterValue(Command(['xacro ', urdf_file]), value_type=str). |
| Browser-downloaded zip has stale content | Browser saved a duplicate as file (1).zip (see §4.5). | Verify contents with python3 -m zipfile -l <file>; use the right path on scp. |

## 14. UDP command reference for the motion host

From “Jueying Lite3 Motion Host Communication Interface v1.0.7-0” (DEEP Robotics, 2024-05-15).

### 14.1 Packet format

Simple commands (12 bytes, little-endian):

struct CommandHead {
    uint32_t code;            // command code
    uint32_t paramters_size;  // 0 for simple commands
    uint32_t type;            // 0 for simple commands
};

Complex commands (used by motion_sender for speed):

struct Command {
    CommandHead head;            // type = 1
    uint32_t data[kDataSize];    // payload (e.g. double-precision speed)
};

UDP port: 43893. Target IP from the dog itself: 127.0.0.1. From a host on segment 1: 192.168.1.120.

### 14.2 Common simple command codes

| Command | Code | Spec section |
| --- | --- | --- |
| Heartbeat (≥2 Hz to maintain session) | 0x21040001 | 1.2.1 |
| Navigation mode (enables /cmd_vel) | 0x21010C03 | 1.2.7 |
| Manual mode (joystick takeover) | 0x21010C02 | 1.2.7 |
| Move mode (axis commands move robot) | 0x21010D06 | 1.2.4 |
| Pose mode (axis commands rotate body) | 0x21010D05 | 1.2.4 |
| Stand/sit toggle | 0x21010202 | 1.2.2 |
| Soft emergency stop | 0x21020C0E | 1.2.2 |
| Reset to zero | 0x21010C05 | 1.2.2 |

### 14.3 Speed commands (complex, used by motion_sender)

These are sent at ≥20 Hz by motion_sender on the dog as it forwards /cmd_vel. You normally won’t send them yourself, but Appendix A.3 has a working example.

| Command | Code | Type | Range |
| --- | --- | --- | --- |
| Linear velocity X (m/s, forward) | 0x0140 | complex | [-1.0, 1.0] |
| Linear velocity Y (m/s, lateral) | 0x0145 | complex | [-0.5, 0.5] |
| Angular velocity Z (rad/s, yaw) | 0x0141 | complex | [-1.5, 1.5] |

## 15. Frame IDs and TF tree

map
 └── odom              (slam_toolbox)
      └── base_link    (odom_bridge_node, from /leg_odom2)
           └── laser   (robot_state_publisher from URDF)

Frames:

- map: SLAM-managed world frame. Subject to discrete loop-closure jumps.

- odom: Continuous odometric frame. Published by odom_bridge_node. Drifts but never jumps.

- base_link: Robot body frame, conventionally at the body centerpoint.

- laser: RPLIDAR A2M8 sensor frame.

Measured base_link → laser static offset: x = −0.060 m, y = 0.000 m, z = 0.097 m (LiDAR is 6 cm behind body center, 9.7 cm above). Defined in urdf/lite3_minimal.urdf.xacro. Update this if the LiDAR is remounted.

## 16. Open-loop orbit drift

At 0.75 m radius with default orbit settings, radius drift over a full 360° is typically 10–20 cm.

Sources of drift: - Open-loop velocity commands — no feedback from /leg_odom2 to correct radius - Trapezoidal envelope has discrete ramp transitions that the dog smooths via its own dynamics - Lateral and yaw aren’t perfectly orthogonal in the dog’s gait

Implications: - First-pass splats work but reconstruct fuzzier at the perimeter than ideal - Visual hand-aiming the dog at the start is worth doing - Closed-loop mitigation (radius correction using /leg_odom2) is a planned improvement

## 17. External references

- Jueying Lite3 Motion Host Communication Interface v1.0.7-0 (DEEP Robotics, 2024-05-15). PDF in project docs. Sections 1.2.7 (control modes), 1.2.12 (speed commands), 1.1 (packet format).

- Lite3_MotionSDK on GitHub. For motion host IP determination and reference C++ client.

- ROS2 Foxy documentation: https://docs.ros.org/en/foxy/

- Nav2 (Foxy) documentation: https://navigation.ros.org/

- Nerfstudio documentation: https://docs.nerf.studio/

- splatfacto method: linked from Nerfstudio docs.

# Appendix A — Exact dog-side mode scripts

Run these on the dog itself. These are the scripts that worked. Do not run them from the NUC container — the motion host is reachable only from the dog, at 127.0.0.1:43893.

## A.1 Put dog into navigation + move mode

python3 - <<'PY'
import socket
import struct
import time

HOST = "127.0.0.1"
PORT = 43893
HEARTBEAT = 0x21040001
NAVIGATION_MODE = 0x21010C03
MOVE_MODE = 0x21010D06

def simple(code):
    return struct.pack("<III", code, 0, 0)

sock = socket.socket(socket.AF_INET, socket.SOCK_DGRAM)

def send(code, name):
    payload = simple(code)
    print(f"sent {name}: code=0x{code:08X}, bytes={' '.join(f'{b:02x}' for b in payload)}")
    sock.sendto(payload, (HOST, PORT))

for _ in range(5):
    send(HEARTBEAT, "heartbeat")
    time.sleep(0.2)

for _ in range(5):
    send(NAVIGATION_MODE, "NAVIGATION / automatic mode")
    time.sleep(0.2)

for _ in range(5):
    send(MOVE_MODE, "MOVE mode")
    time.sleep(0.2)

for _ in range(5):
    send(HEARTBEAT, "heartbeat")
    time.sleep(0.2)

sock.close()
print("Dog should now be in automatic/navigation + move mode.")
PY

## A.2 Return dog to manual / controller mode

python3 - <<'PY'
import socket
import struct
import time

HOST = "127.0.0.1"
PORT = 43893
HEARTBEAT = 0x21040001
MANUAL_MODE = 0x21010C02

def simple(code):
    return struct.pack("<III", code, 0, 0)

sock = socket.socket(socket.AF_INET, socket.SOCK_DGRAM)

for _ in range(3):
    sock.sendto(simple(HEARTBEAT), (HOST, PORT))
    time.sleep(0.2)

for _ in range(5):
    payload = simple(MANUAL_MODE)
    print(f"sent MANUAL mode: {' '.join(f'{b:02x}' for b in payload)}")
    sock.sendto(payload, (HOST, PORT))
    time.sleep(0.2)

sock.close()
print("Dog should now be back in manual/controller mode.")
PY

## A.3 Direct UDP forward speed test that moved the dog

Use only in a clear area. This sends SPEED_X directly to the dog motion host after switching modes.

python3 - <<'PY'
import socket
import struct
import time

HOST = "127.0.0.1"
PORT = 43893
HEARTBEAT = 0x21040001
NAVIGATION_MODE = 0x21010C03
MOVE_MODE = 0x21010D06
SPEED_X = 0x0140

def simple(code, value=0):
    return struct.pack("<III", code, value, 0)

def complex_double(code, value):
    return struct.pack("<III", code, 8, 1) + struct.pack("<d", float(value))

sock = socket.socket(socket.AF_INET, socket.SOCK_DGRAM)

for _ in range(5):
    sock.sendto(simple(HEARTBEAT), (HOST, PORT))
    time.sleep(0.2)

for _ in range(5):
    sock.sendto(simple(NAVIGATION_MODE), (HOST, PORT))
    time.sleep(0.2)

for _ in range(5):
    sock.sendto(simple(MOVE_MODE), (HOST, PORT))
    time.sleep(0.2)

t0 = time.time()
while time.time() - t0 < 1.0:
    sock.sendto(simple(HEARTBEAT), (HOST, PORT))
    sock.sendto(complex_double(SPEED_X, 0.04), (HOST, PORT))
    time.sleep(0.05)

for _ in range(10):
    sock.sendto(complex_double(SPEED_X, 0.0), (HOST, PORT))
    time.sleep(0.05)

sock.close()
print("Done.")
PY

Exact commands for Ayaan/Ahmed lab setup. Use commands exactly unless a line says to substitute a value.